UAV SIMULATOR

PHASE 2 — ADVANCED MULTIPHYSICS

Dev 2 (“Dev A”) — Testing & Debugging Guide

Companion to: Phase 2 Dev A (FEA) Build Guide. Tools: your FEA solver, Python, RustRover.

Verifies FEA case correctness, the structural surrogate, its provenance metadata, and structural.rs — to Tier F rigor.

# 1. How to Use This Document

Your Build Guide told you how to build a structural surrogate. This document tells you how to know its numbers are actually correct — not just “it ran without an error,” but “it matches hand-calculable cases and known physical limits.” Per the Standards & Safety Classification Framework, structural.rs is Tier F: its output is a safety margin, and a wrong number here is exactly the kind of thing the MIL-STD-882E hazard matrix in that document calls Catastrophic-severity if it ever influenced a real design decision. Every test below traces to a row in TRACEABILITY.md.

- Section 2 sets up testing for your specific FEA tool.

- Section 3 is a quick check after each Build Guide task.

- Section 4 is debugging method plus a symptom → cause table.

- Section 5 is the Master Test List, in four groups: hand-calculable FEA cases, surrogate isolation tests, provenance tests, and full pipeline tests.

- Section 6 is the sign-off checklist.

# 2. Setting Up Your FEA Tool and RustRover for This Work

Whichever tool you chose in Task 1 (CalculiX or Code_Aster), confirm it can output raw stress/deformation numbers in a script-readable format, not just a visual plot — you need to compare specific numbers against hand calculations, not eyeball a color map. On the Rust side, the same RustRover debugging approach from your Phase 1 integrator guide applies directly: breakpoints, the Variables panel, and printing intermediate values before trusting a final combined result.

# 3. Testing Checkpoints — After Each Build Task

| After Task | Quick Check | Expected |
| --- | --- | --- |
| 1 (FEA tool chosen) | Run the tool's own tutorial example | Matches the tutorial's documented expected output |
| 4 (first test case) | Simple downward G-load case, compare against hand beam-bending calculation | Within a few percent — see FEA-B1 below for the precise bar |
| 6 (bootstrap + initial surrogate) | Query the surrogate on a held-back bootstrap point | Produces both a prediction and an uncertainty value |
| 8 (validated surrogate) | Held-out error is an explicit reported number | Not just “looks good” — a specific percentage, documented |
| 9 (export) | Exported model has all 4 required metadata fields | geometry_hash, design_revision, trained_at_utc, trained_by all present |

# 4. General Debugging Method

- Reduce to the simplest possible load case first — one axis, no vibration, a load you can compute by hand.

- If the surrogate's answer looks wrong, check the raw FEA case output for that same condition before assuming the surrogate is at fault — the surrogate can only be as good as the data it was trained on.

- Check units at every boundary — FEA tools vary in whether they expect force in Newtons or another unit by default; a silent unit mismatch produces plausible-looking wrong numbers, the worst kind of bug for a safety margin.

- After any fix, re-run the entire Master Test List, not just the failing test.

### Symptom → Likely Cause

| Symptom | Likely Cause | Check |
| --- | --- | --- |
| Stress values are off by a large, suspicious factor (10x, 1000x) | Unit mismatch (e.g. mm vs m, MPa vs Pa) | Check every unit conversion at the FEA tool's input/output boundary explicitly |
| Safety margin doesn't change at all across very different loads | The surrogate's inputs aren't actually reaching the solver, or training data lacks variation on that axis | Print the exact inputs the training script recorded for several cases and confirm they actually differ |
| Mesh convergence never stabilizes | Element size still too coarse, or a geometry feature (sharp corner) needs local refinement | Halve the element size once more before concluding the physics itself is wrong |
| Provenance metadata missing on export | Training script run without --embed-provenance, or run manually instead of through train-surrogate.yml | Re-run through the automated workflow, which sets this by default |

# 5. Master Test List

## Group A — Build & Environment

### FEA-A1 — Rust project builds cleanly with structural.rs added

Why this test matters: Baseline — nothing else can be trusted otherwise.

How to run it: cargo build --release from physics_core.

Expected result (tolerance is the requirement, not a suggestion): Zero errors, zero unused-variable warnings.

If it fails: An unused parameter often means a G-load/vibration/angle input isn't actually reaching your surrogate query.

Traces to: TRC-FEA-001

## Group B — Hand-Calculable FEA Cases

### FEA-B1 — Simple cantilever bending matches analytical beam theory

Why this test matters: A single arm under a known tip load has an exact, textbook closed-form answer — this is your free-fall-test equivalent, the case that proves the whole solver setup is fundamentally sound before trusting anything more complex.

How to run it: Model one frame arm as a simple cantilever beam, apply a known point load at the tip, and compute the expected tip deflection by hand using standard beam-bending theory (deflection = FL³/3EI).

Expected result (tolerance is the requirement, not a suggestion): FEA-predicted deflection within 5% of the hand-calculated value — the small residual is expected from mesh discretization, not a red flag by itself.

If it fails: If it's off by more than that, check material property units first (Young's modulus is the most common source of a large, silent error), then mesh density.

Traces to: TRC-FEA-002

### FEA-B2 — Doubling the applied load roughly doubles the peak stress (linear regime)

Why this test matters: Within the material's elastic range, stress should scale linearly with load — a cheap, powerful sanity check that doesn't require a textbook formula for your specific geometry.

How to run it: Run the same case at two different load magnitudes, well within the elastic range.

Expected result (tolerance is the requirement, not a suggestion): Peak stress ratio between the two runs matches the load ratio within a few percent.

If it fails: A significant deviation suggests either a nonlinear solver setting was accidentally enabled, or a boundary condition changed between runs.

Traces to: TRC-FEA-003

### FEA-B3 — Mesh convergence: refining the mesh further doesn't meaningfully change the answer

Why this test matters: Proves the FEA setup itself has stabilized on a real answer, independent of any comparison to the real world — required before Task 2's UIUC-style validation step means anything.

How to run it: Re-run FEA-B1's case at progressively finer mesh resolution.

Expected result (tolerance is the requirement, not a suggestion): Peak stress/deflection changes by less than 2% between the two finest resolutions tested.

If it fails: Keep refining, or check for a geometry feature (sharp corner, thin section) that needs local mesh refinement rather than global.

Traces to: TRC-FEA-004

## Group C — Surrogate Isolation Tests

### FEA-C1 — Surrogate produces both a prediction and an uncertainty for a held-out point

Why this test matters: Confirms the GPR fit itself is functioning correctly, isolated from the FFI layer.

How to run it: Query the trained surrogate directly in Python on a bootstrap point deliberately excluded from training.

Expected result (tolerance is the requirement, not a suggestion): A numeric prediction and a numeric, non-zero uncertainty.

If it fails: Confirm the held-out point was actually excluded from the training call, not accidentally included.

Traces to: TRC-FEA-005

### FEA-C2 — Safety margin decreases monotonically as G-load increases (all else held constant)

Why this test matters: This is a physical sanity check the surrogate should obey even though it's a statistical fit, not a hand-derived formula — a real structure gets less safe under more load, never more safe.

How to run it: Query the surrogate across a swept range of G-load values, holding vibration frequency and impact angle fixed.

Expected result (tolerance is the requirement, not a suggestion): Safety margin trends downward as G-load increases, with no large non-physical jumps.

If it fails: If it's non-monotonic, check for insufficient training data density in that region, or a units/sign error in how G-load was recorded during case setup.

Traces to: TRC-FEA-006

### FEA-C3 — Held-out validation error is reported as an explicit, honest number

Why this test matters: “More cases” only proves the surrogate mirrors your own FEA setup — this test is what actually quantifies that, per the Standards Framework's distinction between the two kinds of accuracy.

How to run it: Run the full held-out validation set (never used in training) and compute prediction error against the true FEA result for each.

Expected result (tolerance is the requirement, not a suggestion): A specific, documented error percentage, and a documented statement of exactly which G-load/vibration/angle envelope this validation covers.

If it fails: A surprisingly high error suggests the bootstrap/active-learning set didn't adequately cover this region — add targeted cases there, don't just accept a wide error bar.

Traces to: TRC-FEA-007

## Group D — Provenance & Export

### FEA-D1 — Exported model carries all four required metadata fields

Why this test matters: The Lead Architect's loader will refuse anything missing these — catching it here, before the PR, is faster than debugging a rejected load later.

How to run it: Inspect the exported .onnx file's metadata_props directly (most ONNX tooling can dump this).

Expected result (tolerance is the requirement, not a suggestion): geometry_hash, design_revision, trained_at_utc, and trained_by are all present and non-empty.

If it fails: Confirm training ran through train-surrogate.yml with --embed-provenance, not a manual local run.

Traces to: TRC-FEA-008 (Standards Framework Part II)

### FEA-D2 — Embedded geometry_hash matches the current canonical geometry file

Why this test matters: A mismatch here means the Lead Architect's loader will correctly reject this exact model — better to catch it in your own testing than discover it in a failed integration run.

How to run it: Compute SHA-256 of offline_pipeline/geometry/current_geometry.json yourself, and compare against the model's embedded geometry_hash.

Expected result (tolerance is the requirement, not a suggestion): Exact match.

If it fails: If the geometry changed after this training run started, retrain against the current file — this isn't a bug, it's the gate doing its job.

Traces to: TRC-FEA-009

## Group E — Full Pipeline Integration (structural.rs)

### FEA-E1 — structural.rs query reaches the loaded surrogate and returns a value

Why this test matters: Confirms the wiring from FFI down to your specific module, not just the surrogate loader in isolation.

How to run it: Trigger a physics step with a known structural condition and read the returned safety margin.

Expected result (tolerance is the requirement, not a suggestion): A non-zero, sensible safety margin matching what the surrogate itself returns for that same input, queried directly.

If it fails: Check input ordering into the query — a silently transposed G-load/vibration/angle input is a common, hard-to-notice bug.

Traces to: TRC-FEA-010

### FEA-E2 — An out-of-envelope structural condition is flagged, not silently trusted

Why this test matters: Matches the same requirement Dev B's aero surrogate has — consistency across both surrogate types matters for anyone maintaining this code later.

How to run it: Query with a G-load value well outside your validated sweep range.

Expected result (tolerance is the requirement, not a suggestion): in_validated_envelope is false.

If it fails: Confirm the envelope check uses the actual documented range from FEA-C3, not a stale or guessed range.

Traces to: TRC-FEA-011

### FEA-E3 — Extreme (but physically plausible) G-load doesn't produce NaN

Why this test matters: Numerical stability at the edges matters even outside the validated envelope — an out-of-envelope answer should be flagged as untrustworthy, not literally undefined.

How to run it: Query with a very high G-load, several times normal flight loading.

Expected result (tolerance is the requirement, not a suggestion): A numeric result (even if flagged out-of-envelope), never NaN or Infinity.

If it fails: Check for a division that could have a near-zero denominator at extreme inputs.

Traces to: TRC-FEA-012

# 6. Final Sign-Off Checklist

structural.rs is Tier F. Every box below must be checked in this session, and the PR requires a reviewer who is not you.

- All Group A tests passed (A1)

- All Group B tests passed (B1–B3) — hand-calculable case matched, mesh convergence confirmed

- All Group C tests passed (C1–C3) — monotonicity and honest error reporting confirmed

- All Group D tests passed (D1–D2) — provenance metadata verified against the live geometry file

- All Group E tests passed (E1–E3)

- TRACEABILITY.md updated with every TRC-FEA-### row above, marked PASS

- Independent reviewer (not you) has approved the PR