UAV SIMULATOR

PHASE 2 — ADVANCED MULTIPHYSICS

Dev 4 (“Dev C”) — Testing & Debugging Guide

Companion to: Phase 2 Dev C (Thermal/Telemetry) Build Guide. Tools: RustRover, Rider, UE5 Editor.

Verifies the thermal/electrical model, cooling integration, and every new OSD warning state — including the two-failure-state distinction.

# 1. How to Use This Document

Your Build Guide told you how to build the thermal/electrical module and extend the OSD. This document tells you how to know both are actually correct. thermal.rs is Tier F per the Standards Framework (its output is meant to reflect real physical limits, not just a display number), while the OSD extension itself is Tier V (it displays state, it doesn't compute anything) — so this document has two different rigor bars in it, and Section 5 is split accordingly. Every Tier F test below traces to a row in TRACEABILITY.md; the Tier V (OSD) tests don't need that same traceability discipline, but they absolutely need to actually be run.

- Section 2 sets up testing across RustRover, Rider, and the UE5 Editor.

- Section 3 is a quick check after each Build Guide task.

- Section 4 is debugging method plus a symptom → cause table.

- Section 5 is the Master Test List: electrical/thermal math (Tier F), then OSD display correctness (Tier V).

- Section 6 is the sign-off checklist.

# 2. Setting Up for This Work

For the Rust half, the same RustRover approach from Phase 1 applies — breakpoints and printed intermediate values, especially useful for confirming current/heat calculations at each stage of the electrical chain before trusting the final temperature. For the C++/OSD half, you'll need a way to force specific test conditions (a known throttle level, a forced out-of-envelope query, a forced GeometryMismatch) rather than waiting for them to occur naturally during a normal flight — coordinate with the Lead Architect and Dev A/Dev B on the simplest way to trigger each one deliberately for testing.

# 3. Testing Checkpoints — After Each Build Task

| After Task | Quick Check | Expected |
| --- | --- | --- |
| 3 (electrical chain) | Known current + known resistance, compute I²R by hand | Matches your function's output — see THM-B1 below for the exact bar |
| 4 (cooling) | Higher airflow value produces a lower steady-state temperature, all else equal | Temperature decreases monotonically as airflow increases |
| 5 (OSD extension) | New panel fields show real, changing values during a live flight | Not static placeholders — actual live data |
| 6 (scaling/color) | New panel doesn't visually overlap or obscure the Phase 1 force-arrow readout | Both fully readable at once |

# 4. General Debugging Method

- For a wrong temperature, isolate current calculation, then heat calculation, then cooling, in that order, before assuming the bug is in the final combined result.

- For an OSD display bug, confirm the underlying FFI struct field actually holds the expected value (check it in the Rust debugger first) before assuming the C++ display logic is at fault — half of “display bugs” are actually upstream data bugs.

- After any fix, re-run the entire Master Test List, not just the failing test.

### Symptom → Likely Cause

| Symptom | Likely Cause | Check |
| --- | --- | --- |
| Motor temperature climbs without bound | Cooling term missing or using the wrong sign | Confirm cooling is actually subtracted from generated heat, not added |
| Temperature is identical regardless of throttle | Current calculation not actually using the throttle input | Print the computed current at several different throttle levels |
| Envelope warning and GeometryMismatch warning look identical on screen | Both routed to the same generic warning banner instead of distinct states | Give GeometryMismatch its own visual state per your Build Guide Task 5 |
| OSD shows a stale value that doesn't update | Reading from a cached DebugTelemetry snapshot instead of the current tick's | Confirm the read happens inside TickComponent every frame, not once at startup |

# 5. Master Test List

## Group A — Build (Tier F + Tier V baseline)

### THM-A1 — Rust and UE5 projects both build cleanly

Why this test matters: Baseline for everything downstream.

How to run it: cargo build --release, then rebuild the UE5 project in Rider.

Expected result (tolerance is the requirement, not a suggestion): Zero errors on both sides.

If it fails: Check for a mismatched field name between thermal.rs's output struct and what the OSD reads.

Traces to: TRC-THM-001

## Group B — Electrical/Thermal Math (Tier F)

### THM-B1 — I²R heat calculation matches a hand calculation exactly

Why this test matters: The most basic law your whole thermal model rests on — if this simple case is wrong, everything built on top is wrong too, just less obviously.

How to run it: Call your heat-generation function with a known current (for example, 5 amps) and a known resistance (for example, 0.1 ohms). Compute expected heat by hand: I²R = 25 × 0.1 = 2.5 watts.

Expected result (tolerance is the requirement, not a suggestion): Function returns 2.5 watts, within a very small rounding tolerance.

If it fails: Check you're squaring current, not doubling it, and that resistance units match what your constants file actually documents.

Traces to: TRC-THM-002

### THM-B2 — Zero throttle produces zero current and zero generated heat

Why this test matters: Simplest possible boundary condition — no commanded power, no heat.

How to run it: Call the electrical chain function with throttle = 0.0.

Expected result (tolerance is the requirement, not a suggestion): Current and generated heat both exactly zero.

If it fails: Check for a nonzero baseline/idle term that shouldn't be there yet, or a constant offset bug.

Traces to: TRC-THM-003

### THM-B3 — Higher airflow monotonically reduces steady-state temperature, all else held constant

Why this test matters: A basic physical sanity check on the cooling term — more moving air should never make a component hotter.

How to run it: Compute steady-state temperature at several different airflow values, holding generated heat constant.

Expected result (tolerance is the requirement, not a suggestion): Temperature decreases (or stays flat at a physical floor) as airflow increases — never increases.

If it fails: Check the sign of the cooling term in your combining equation.

Traces to: TRC-THM-004

### THM-B4 — Extreme (but physically plausible) throttle and airflow values never produce NaN

Why this test matters: Numerical stability at the edges, the same standard your Phase 1 mixer was held to.

How to run it: Call the full thermal chain at maximum throttle and at both very low and very high airflow values.

Expected result (tolerance is the requirement, not a suggestion): A numeric result in every case, never NaN or Infinity.

If it fails: Check for a division that could have a near-zero denominator at an airflow extreme.

Traces to: TRC-THM-005

### THM-E1 — Full pipeline: thermal module correctly reads Dev B's airflow output for the current flight condition

Why this test matters: Confirms actual wiring, not just isolated math — a mismatched unit or stale reference here silently produces a plausible-looking wrong temperature.

How to run it: Trigger a physics step at a known flight condition and confirm the airflow value your module receives matches what Dev B's surrogate/formula independently reports for that same condition.

Expected result (tolerance is the requirement, not a suggestion): Matching values.

If it fails: Coordinate with Dev B to confirm you're both using the exact same units and the exact same struct field, not two similarly-named ones.

Traces to: TRC-THM-006

## Group C — OSD Display Correctness (Tier V)

### THM-C1 — Motor temperature and structural safety margin display real, live-updating values

Why this test matters: The whole point of the OSD is to reflect actual current state, not a static placeholder.

How to run it: Watch the OSD panel during a live simulated flight while deliberately varying throttle.

Expected result (tolerance is the requirement, not a suggestion): Displayed temperature visibly changes in response to throttle changes, within one physics tick's delay.

If it fails: Confirm the read happens inside TickComponent every frame, not cached from an earlier tick.

### THM-C2 — Envelope warning appears when, and only when, in_validated_envelope is false

Why this test matters: This is what turns Dev A and Dev B's careful validation work into something the team can actually see and trust live.

How to run it: Deliberately fly to a condition known to be outside the trained envelope (coordinate with Dev B/Dev A on how to force this), then return to a normal condition.

Expected result (tolerance is the requirement, not a suggestion): Warning appears when out of envelope, disappears when back in envelope — no stuck or flickering state.

If it fails: Confirm the read is per-tick, not latched from the first time it was ever triggered.

### THM-C3 — GeometryMismatch produces a visually distinct warning from the envelope warning

Why this test matters: Per your Build Guide Section 5.6, these are two different problems and collapsing them into one banner hides the more urgent one.

How to run it: Force a GeometryMismatch (coordinate with the Lead Architect on the simplest way — loading a deliberately mismatched test model is one option) and compare the resulting banner against the envelope-warning banner from THM-C2.

Expected result (tolerance is the requirement, not a suggestion): Different color and/or different text — unmistakably two different states, not a shared generic “something's wrong” banner.

If it fails: Revisit Build Guide Task 5 — confirm you're reading the specific GeometryMismatch status code, not reusing the envelope flag's display logic.

### THM-C4 — New panel elements don't obscure or crowd the Phase 1 force-arrow readout

Why this test matters: The new panel is an addition, not a replacement — the original telemetry still matters.

How to run it: Visually inspect the full OSD during a flight with both new and original elements active.

Expected result (tolerance is the requirement, not a suggestion): Both fully readable simultaneously, no overlapping text or arrows.

If it fails: Adjust panel layout/positioning constants — this is a layout fix, not a data fix.

# 6. Final Sign-Off Checklist

thermal.rs is Tier F — Group B and E1 require an independent reviewer and a TRACEABILITY.md update. The OSD extension is Tier V — Group C needs a normal review, but every test in it still needs to actually be run and confirmed, not assumed from the code looking right.

- All Group A tests passed (A1)

- All Group B tests passed (B1–B4, E1) — hand-calculated heat, monotonic cooling, and correct airflow wiring all confirmed

- All Group C tests passed (C1–C4) — both new warning states verified as visually distinct, live-updating, and non-overlapping

- TRACEABILITY.md updated with every TRC-THM-### row above, marked PASS

- Independent reviewer (not you) has approved the Tier F portion of the PR